NETWORK LAB  -  PROJECT 01

Setting Up an Internal Network

Cisco Packet Tracer  -  DHCP  -  Static Addressing  -  Routing

Author:  Amirali Gholian

amiraligholian.ir  -  Network Lab

Project file: downloads/p1_network.pkt

1.  Project Overview

This project builds a small office / lab internal network from scratch inside Cisco Packet Tracer. The goal is to show the everyday tasks a junior network administrator actually performs: assigning IP addresses, configuring DHCP on a dedicated server, mixing DHCP and static clients on the same segment, putting a default route on the edge router, and verifying connectivity end-to-end.

The same workspace is shown in both Logical and Physical views. The report therefore covers both the addressing / routing side and the physical layout - where the rack lives, how the patch panels break out the cabling, and where the end devices sit on the desks.

1.1  What This Network Is

One internal LAN (192.168.10.0 / 24) hosting end users and one server.

One WAN uplink to a simulated ISP so the LAN can reach the outside world.

A mix of DHCP clients and at least one static host on the same segment.

Both Logical and Physical views configured and verified.

1.2  Topology at a Glance

1 x Cisco 2960-24TT  -  Layer-2 access switch.

1 x Cisco 1941  -  edge router, the LAN default gateway.

1 x Cisco 2911  -  simulated upstream / ISP router.

1 x Server-PT  -  combined DHCP and static service server.

2 x DHCP clients  -  PC0 and PC1.

1 x static client  -  PC2 (IP is set by hand to demonstrate non-DHCP hosts).

1.3  IP Addressing Plan

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Edge Router (1941) | G0/0 (LAN) | 192.168.10.1 | 255.255.255.0 | - |
| Edge Router (1941) | G0/1 (WAN) | 10.0.0.1 | 255.255.255.252 | - |
| Upstream Router (2911) | G0/0 (WAN) | 10.0.0.2 | 255.255.255.252 | - |
| Upstream Router (2911) | G0/1 (ISP) | 203.0.113.1 | 255.255.255.252 | - |
| DHCP / Static Server | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC0 (DHCP) | NIC | 192.168.10.101 | 255.255.255.0 | 192.168.10.1 |
| PC1 (DHCP) | NIC | 192.168.10.102 | 255.255.255.0 | 192.168.10.1 |
| PC2 (Static) | NIC | 192.168.10.50 | 255.255.255.0 | 192.168.10.1 |

Why this layout?  The /24 LAN keeps the addressing simple. The /30 WAN link gives just two usable addresses - exactly what a point-to-point ISP uplink needs. PC2's static address is inside the DHCP excluded range, so there is zero risk of duplicate IPs.

2.  Switch Configuration

The 2960-24TT acts as a pure Layer-2 access switch. All ports stay in VLAN 1 - the switch does not route. The only meaningful configuration is a hostname, a banner and a console password so the device is identifiable and locked down.

Switch>  enable

Switch#  configure terminal

Switch(config)#  hostname SW-Internal

SW-Internal(config)#  banner motd # Internal Network - Authorized Access Only #

SW-Internal(config)#  line console 0

SW-Internal(config-line)#  logging synchronous

SW-Internal(config-line)#  password Console123

SW-Internal(config-line)#  login

SW-Internal(config-line)#  exit

SW-Internal(config)#  service password-encryption

SW-Internal(config)#  end

SW-Internal#  write memory

All PC ports use straight-through copper cables. The uplink from the switch to the edge router uses Fa0/1 on the switch and G0/0 on the router. The DHCP server connects to Fa0/2.

3.  DHCP Server Configuration

A dedicated server (192.168.10.10) runs the DHCP service for the whole /24 segment. A single pool serves every dynamic client. Reserved blocks are excluded so that the router's gateway address, the static PC2, and a small management range are never handed out by mistake.

DHCP_Server>  enable

DHCP_Server#  configure terminal

DHCP_Server(config)#  ip dhcp excluded-address 192.168.10.1  192.168.10.20

DHCP_Server(config)#  ip dhcp excluded-address 192.168.10.50 192.168.10.50

DHCP_Server(config)#  ip dhcp pool LAN-POOL

DHCP_Server(dhcp-config)#  network 192.168.10.0 255.255.255.0

DHCP_Server(dhcp-config)#  default-router 192.168.10.1

DHCP_Server(dhcp-config)#  dns-server 192.168.10.1

DHCP_Server(dhcp-config)#  lease 7

DHCP_Server(dhcp-config)#  end

DHCP_Server#  write memory

The excluded ranges reserve 192.168.10.1 for the router, 192.168.10.50 for the static PC2, and the 10-20 block for future static devices (printers, printers, APs, etc.).

4.  Edge Router  (Cisco 1941)

The 1941 is the heart of the network. It carries the LAN default gateway, terminates the WAN link to the upstream router, and runs a single static default route so any non-LAN traffic leaves the network through 10.0.0.2.

Router>  enable

Router#  configure terminal

Router(config)#  hostname EdgeRouter

EdgeRouter(config)#  interface GigabitEthernet0/0

EdgeRouter(config-if)#  description ** LAN UPLINK **

EdgeRouter(config-if)#  ip address 192.168.10.1 255.255.255.0

EdgeRouter(config-if)#  no shutdown

EdgeRouter(config-if)#  exit

EdgeRouter(config)#  interface GigabitEthernet0/1

EdgeRouter(config-if)#  description ** WAN UPLINK **

EdgeRouter(config-if)#  ip address 10.0.0.1 255.255.255.252

EdgeRouter(config-if)#  no shutdown

EdgeRouter(config-if)#  exit

EdgeRouter(config)#  ip route 0.0.0.0 0.0.0.0 10.0.0.2

EdgeRouter(config)#  banner motd # Edge Router - Internal Network #

EdgeRouter(config)#  enable secret Class123

EdgeRouter(config)#  line console 0

EdgeRouter(config-line)#  password Console123

EdgeRouter(config-line)#  login

EdgeRouter(config-line)#  exit

EdgeRouter(config)#  service password-encryption

EdgeRouter(config)#  end

EdgeRouter#  write memory

4.1  Verifying the Edge Router

Two commands are enough to confirm everything is up:

EdgeRouter#  show ip interface brief

Interface              IP-Address      OK? Method Status   Protocol

GigabitEthernet0/0     192.168.10.1    YES manual up        up

GigabitEthernet0/1     10.0.0.1        YES manual up        up

EdgeRouter#  show ip route

Codes: C - connected, S - static, ...

Gateway of last resort is 10.0.0.2 to network 0.0.0.0

S*    0.0.0.0/0 [1/0] via 10.0.0.2

C     10.0.0.0/30 is directly connected, GigabitEthernet0/1

C     192.168.10.0/24 is directly connected, GigabitEthernet0/0

5.  Upstream Router  (Cisco 2911)

The 2911 represents the ISP side of the link. It owns the WAN side of the /30, the loopback-style /30 on the simulated Internet side, and a single static return route back to the LAN so replies can find their way home.

Router>  enable

Router#  configure terminal

Router(config)#  hostname Upstream

Upstream(config)#  interface GigabitEthernet0/0

Upstream(config-if)#  description ** LINK TO EDGE ROUTER **

Upstream(config-if)#  ip address 10.0.0.2 255.255.255.252

Upstream(config-if)#  no shutdown

Upstream(config-if)#  exit

Upstream(config)#  interface GigabitEthernet0/1

Upstream(config-if)#  description ** SIMULATED INTERNET **

Upstream(config-if)#  ip address 203.0.113.1 255.255.255.252

Upstream(config-if)#  no shutdown

Upstream(config-if)#  exit

Upstream(config)#  ip route 192.168.10.0 255.255.255.0 10.0.0.1

Upstream(config)#  end

Upstream#  write memory

6.  End-Device Configuration

6.1  DHCP clients  -  PC0, PC1

On each PC:

Open Desktop > IP Configuration.

Select the DHCP radio button.

Click the renew button. The client sends DHCPDISCOVER, receives a DHCPOFFER and binds the lease.

After the lease is bound, ipconfig / all confirms the assigned IP (192.168.10.101 for PC0 and 192.168.10.102 for PC1), mask 255.255.255.0, gateway 192.168.10.1, and DNS 192.168.10.1.

6.2  Static client  -  PC2

PC2 is configured by hand to show how a server, printer or any fixed device is added to the same network without depending on DHCP.

| Setting | Value |
| --- | --- |
| IP address | 192.168.10.50 |
| Subnet mask | 255.255.255.0 |
| Default gateway | 192.168.10.1 |
| DNS server | 192.168.10.1 |

Why is the address safe?  192.168.10.50 falls inside the excluded range on the DHCP server, so the address can never be handed out to a DHCP client. No risk of duplicates.

7.  Connectivity Tests

All tests are run from the Command Prompt of the relevant device.

7.1  LAN-side tests  -  from PC0

PC0>  ping 192.168.10.1          (default gateway)

PC0>  ping 192.168.10.10         (DHCP server)

PC0>  ping 192.168.10.50         (static PC2)

PC0>  ping 192.168.10.102        (other DHCP client)

All four replies come back with 0% loss. This proves that Layer-2 switching, ARP and the LAN-side interface of the router are all functioning correctly.

7.2  WAN and Internet-side tests  -  from PC0

PC0>  ping 10.0.0.1              (edge router LAN ip - sanity)

PC0>  ping 10.0.0.2              (upstream router WAN ip)

PC0>  ping 203.0.113.1           (simulated Internet)

These pings confirm that the default route 0.0.0.0/0 via 10.0.0.2 is in the routing table and that packets are forwarded out the G0/1 interface of the edge router.

7.3  Return-path test  -  from the upstream router

Upstream#  ping 192.168.10.1     (2911  ->  edge router)

Upstream#  ping 192.168.10.50    (2911  ->  static client)

Both succeed only because the static route 192.168.10.0/24 via 10.0.0.1 is configured on the 2911. Without it the upstream router has no way to send replies back to the LAN and pings time out - the classic asymmetric-routing failure.

8.  Common Issues and Fixes

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| PC0 stuck on 169.254.x.x | DHCP server unreachable or wrong default-router on the pool | Re-check 'default-router 192.168.10.1' on the DHCP pool |
| Ping to 192.168.10.1 fails | Switch port in the wrong VLAN or router interface down | Run 'no shutdown' on G0/0; confirm the port is in VLAN 1 |
| Ping to 10.0.0.2 fails | Default route missing on the edge router | Add 'ip route 0.0.0.0 0.0.0.0 10.0.0.2' |
| LAN can reach the Internet but the upstream router cannot reach the LAN | No return route on the upstream router | Add 'ip route 192.168.10.0 255.255.255.0 10.0.0.1' on the 2911 |

9.  Physical Workspace

The same network is shown in Physical view to demonstrate how a real office or lab is wired:

Main Wiring Closet - the 2960-24TT, the 1941 edge router, the DHCP / static server and the primary patch panel.

Wiring Closet - secondary patch panel used to break out cabling to the desks.

Three desks with the client PCs, each connected back to the patch panel with straight-through copper.

All cabling is straight-through copper. The router-to-switch link, the server link and the upstream link are GigabitEthernet; the PC links are FastEthernet - the mix you find in most real deployments.

10.  Conclusion

Project 01 demonstrates a complete, end-to-end working internal network: a Layer-2 access switch, a router providing the default gateway, a DHCP server handing out addresses, one static host, and a return path to a simulated upstream network. Every step is verified with ping, and the troubleshooting table lists the most common beginner mistakes and how to fix them. The same baseline scales naturally to any small office, classroom or home-lab.

Amirali Gholian  -  Network Lab  -  Cisco Packet Tracer