NETWORK LAB  -  PROJECT 02

University LAN Scenario

Cisco Packet Tracer  -  Multi-Router  -  Static Routing  -  DHCP  -  ACLs

Author:  Amirali Gholian

amiraligholian.ir  -  Network Lab

Project file: downloads/p2_lan_university.pkt

1.  Project Overview

This project models a small university campus. The campus is divided into independent departments, each with its own router, switch and end hosts. The departmental routers talk to each other through a central backbone router, and static routes are configured on every device so that any department can reach any other department.

ACLs are added on the edge of one department to show how traffic between faculties can be restricted (for example, blocking ICMP echo between two departments while still allowing normal data traffic).

1.1  Topology at a Glance

3 x Cisco 2911 ISR  -  R-DepartmentA, R-DepartmentB and R-Backbone.

2 x Cisco 2960-24TT  -  one access switch per active department.

2 x end-host PCs  -  one student PC per department.

Point-to-point serial links between every router pair.

Static routes on every router for full reachability inside the 10.10.0.0/16 summary.

1.2  IP Addressing Plan

| Device / Link | Interface | IP Address | Subnet |
| --- | --- | --- | --- |
| R-DepartmentA - LAN | G0/0 | 10.10.10.1 | 255.255.255.0 (/24) |
| R-DepartmentA - Backbone | S0/0/0 | 10.10.20.1 | 255.255.255.252 (/30) |
| R-DepartmentB - LAN | G0/0 | 10.10.30.1 | 255.255.255.0 (/24) |
| R-DepartmentB - Backbone | S0/0/0 | 10.10.20.5 | 255.255.255.252 (/30) |
| R-Backbone - Dept A | S0/0/0 | 10.10.20.2 | 255.255.255.252 (/30) |
| R-Backbone - Dept B | S0/0/1 | 10.10.20.6 | 255.255.255.252 (/30) |
| PC0 (Mechanicals) | NIC | 10.10.10.10 | 255.255.255.0 |
| PC1 (Computer) | NIC | 10.10.30.10 | 255.255.255.0 |

Why three /30s?  Each router-to-router link is a point-to-point serial, so a /30 with two usable hosts is the most efficient choice. The two departmental LANs use /24s so they have room to grow.

2.  R-DepartmentA  -  Mechanicals Faculty

Department A's router terminates the Mechanicals LAN and runs a serial uplink to the backbone router. It also serves as the DHCP server for its own department.

Router>  enable

Router#  configure terminal

Router(config)#  hostname R-DepartmentA

R-DepartmentA(config)#  interface GigabitEthernet0/0

R-DepartmentA(config-if)#  description ** LAN - Mechanicals Department **

R-DepartmentA(config-if)#  ip address 10.10.10.1 255.255.255.0

R-DepartmentA(config-if)#  no shutdown

R-DepartmentA(config-if)#  exit

R-DepartmentA(config)#  interface Serial0/0/0

R-DepartmentA(config-if)#  description ** UPLINK TO R-BACKBONE **

R-DepartmentA(config-if)#  ip address 10.10.20.1 255.255.255.252

R-DepartmentA(config-if)#  clock rate 64000

R-DepartmentA(config-if)#  no shutdown

R-DepartmentA(config-if)#  exit

R-DepartmentA(config)#  ip route 10.10.30.0 255.255.255.0 10.10.20.2

R-DepartmentA(config)#  banner motd # Department A - Mechanicals #

R-DepartmentA(config)#  enable secret Class123

R-DepartmentA(config)#  line console 0

R-DepartmentA(config-line)#  password Console123

R-DepartmentA(config-line)#  login

R-DepartmentA(config-line)#  exit

R-DepartmentA(config)#  service password-encryption

R-DepartmentA(config)#  end

R-DepartmentA#  write memory

3.  R-DepartmentB  -  Computer Faculty

Department B is configured in exactly the same way as Department A, just with a different LAN subnet and a different /30 link.

Router>  enable

Router#  configure terminal

Router(config)#  hostname R-DepartmentB

R-DepartmentB(config)#  interface GigabitEthernet0/0

R-DepartmentB(config-if)#  description ** LAN - Computer Department **

R-DepartmentB(config-if)#  ip address 10.10.30.1 255.255.255.0

R-DepartmentB(config-if)#  no shutdown

R-DepartmentB(config-if)#  exit

R-DepartmentB(config)#  interface Serial0/0/0

R-DepartmentB(config-if)#  description ** UPLINK TO R-BACKBONE **

R-DepartmentB(config-if)#  ip address 10.10.20.5 255.255.255.252

R-DepartmentB(config-if)#  no shutdown

R-DepartmentB(config-if)#  exit

R-DepartmentB(config)#  ip route 10.10.10.0 255.255.255.0 10.10.20.6

R-DepartmentB(config)#  banner motd # Department B - Computer #

R-DepartmentB(config)#  enable secret Class123

R-DepartmentB(config)#  line console 0

R-DepartmentB(config-line)#  password Console123

R-DepartmentB(config-line)#  login

R-DepartmentB(config-line)#  exit

R-DepartmentB(config)#  service password-encryption

R-DepartmentB(config)#  end

R-DepartmentB#  write memory

4.  R-Backbone

The backbone router sits in the middle of the campus and forwards traffic between the two departments. It owns both ends of the two serial /30s and carries static routes to both departmental LANs.

Router>  enable

Router#  configure terminal

Router(config)#  hostname R-Backbone

R-Backbone(config)#  interface Serial0/0/0

R-Backbone(config-if)#  description ** TO DEPARTMENT A **

R-Backbone(config-if)#  ip address 10.10.20.2 255.255.255.252

R-Backbone(config-if)#  clock rate 64000

R-Backbone(config-if)#  no shutdown

R-Backbone(config-if)#  exit

R-Backbone(config)#  interface Serial0/0/1

R-Backbone(config-if)#  description ** TO DEPARTMENT B **

R-Backbone(config-if)#  ip address 10.10.20.6 255.255.255.252

R-Backbone(config-if)#  no shutdown

R-Backbone(config-if)#  exit

R-Backbone(config)#  ip route 10.10.10.0 255.255.255.0 10.10.20.1

R-Backbone(config)#  ip route 10.10.30.0 255.255.255.0 10.10.20.5

R-Backbone(config)#  banner motd # Campus Backbone - Do not edit #

R-Backbone(config)#  enable secret Class123

R-Backbone(config)#  line console 0

R-Backbone(config-line)#  password Console123

R-Backbone(config-line)#  login

R-Backbone(config-line)#  exit

R-Backbone(config)#  service password-encryption

R-Backbone(config)#  end

R-Backbone#  write memory

5.  ACL  -  Restricting ICMP Between Departments

A standard numbered ACL is applied to Department A's LAN interface so that ICMP echo-requests coming from Department B are silently dropped, while every other protocol is still allowed through. This mimics a common real-world policy: students in one faculty can use services in another, but they cannot ping them.

R-DepartmentA(config)#  access-list 100 deny   icmp 10.10.30.0 0.0.0.255 10.10.10.0 0.0.0.255 echo

R-DepartmentA(config)#  access-list 100 permit ip any any

R-DepartmentA(config)#  interface GigabitEthernet0/0

R-DepartmentA(config-if)#  ip access-group 100 in

R-DepartmentA(config-if)#  end

R-DepartmentA#  write memory

Verification:  show ip access-lists lists both rules with hit counters; show ip interface GigabitEthernet0/0 confirms 'Inbound access list is 100'.

6.  DHCP for Each Department

Each departmental router runs its own DHCP pool. This keeps address management local to the department and avoids relying on a central server.

R-DepartmentA(config)#  ip dhcp excluded-address 10.10.10.1  10.10.10.20

R-DepartmentA(config)#  ip dhcp pool DEPT-A-LAN

R-DepartmentA(dhcp-config)#  network 10.10.10.0 255.255.255.0

R-DepartmentA(dhcp-config)#  default-router 10.10.10.1

R-DepartmentA(dhcp-config)#  dns-server 10.10.10.1

R-DepartmentA(dhcp-config)#  lease 7

R-DepartmentA(dhcp-config)#  end

R-DepartmentA#  write memory

The same configuration is repeated on R-DepartmentB with the 10.10.30.0/24 network and 10.10.30.1 as the default-router.

7.  End-Device Configuration

7.1  PC0  -  Department A  (Mechanicals)

| Setting | Value |
| --- | --- |
| IP address | 10.10.10.10 |
| Subnet mask | 255.255.255.0 |
| Default gateway | 10.10.10.1 |
| DNS server | 10.10.10.1 |

7.2  PC1  -  Department B  (Computer)

| Setting | Value |
| --- | --- |
| IP address | 10.10.30.10 |
| Subnet mask | 255.255.255.0 |
| Default gateway | 10.10.30.1 |
| DNS server | 10.10.30.1 |

8.  Connectivity Tests

8.1  Forward path  -  from PC0

PC0>  ping 10.10.10.1         (default gateway  -  must succeed)

PC0>  ping 10.10.30.1         (far-end router LAN  -  must succeed)

PC0>  ping 10.10.30.10        (far-end PC  -  must succeed)

All three pings succeed, which proves both directions of every static route and every serial link are healthy.

8.2  Reverse path  -  from PC1

PC1>  ping 10.10.10.10        (reverse direction)

PC1>  ping 10.10.10.1         (reverse default gateway)

9.  Verifying the ACL

9.1  Blocked ICMP  -  from PC1

PC1>  ping 10.10.10.1         (ICMP to Dept A's router  -  blocked by ACL 100)

PC1>  ping 10.10.10.10        (ICMP to PC0  -  blocked by ACL 100)

Both pings time out, which is exactly what the ACL was designed to do. A TCP-based service (HTTP, traceroute using TCP, or telnet) would still work, demonstrating that the ACL only blocks ICMP echo - not all traffic.

10.  Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Serial interface stuck in 'down / down' | DCE side missing clock rate | Add 'clock rate 64000' on the side with the clock symbol |
| ping 10.10.30.1 from PC0 fails | Static route missing on R-DepartmentA or R-Backbone | Re-check 'ip route' entries on both routers |
| ACL does not appear to block anything | ACL applied to the wrong interface or wrong direction | Re-apply 'ip access-group 100 in' on G0/0 of R-DepartmentA |
| DHCP clients stuck on 169.254.x.x | Pool misconfigured or excluded range too wide | Verify 'network', 'default-router' and 'excluded-address' statements |

11.  Physical Layer

The same scenario is also shown in Physical view. The Corporate Office floor plan contains two wiring closets - 'Wiring Closet' and 'Main Wiring Closet' - each with its own patch panel, switch and router. The serial links between routers are drawn as red cables, the LAN links as black. End devices sit on the desks in their respective faculty rooms.

12.  Conclusion

Project 02 scales the ideas from Project 01 into a multi-router campus: each department has its own subnet, its own router and its own DHCP pool, and a backbone router ties them together with static routes. ACLs add a layer of policy. The whole configuration is small enough to type by hand in a few minutes, but it covers every concept a CCNA-level student is expected to know: router interfaces, static routes, DHCP pools, access lists, and the relationship between Logical and Physical views in Packet Tracer.

Amirali Gholian  -  Network Lab  -  Cisco Packet Tracer