NETWORK LAB  -  PROJECT 03

Connecting to a Google Web Server

Cisco Packet Tracer  -  End-to-End Internet  -  DHCP  -  DNS  -  NAT / PAT  -  HTTPS

Author:  Amirali Gholian

amiraligholian.ir  -  Network Lab

Project file: downloads/p3_google_webserver.pkt

1.  Project Overview

This project simulates a small corporate office whose users open a browser and reach https://www.google.com. The full chain of technologies that makes this work in the real world is reproduced inside Cisco Packet Tracer: an internal LAN with three client PCs, an access switch, an edge router running DHCP for the LAN and PAT (Port Address Translation) for outbound traffic, an upstream router simulating the Internet, and two servers - one acting as the public Google DNS (8.8.8.8) and one acting as the Google Web Server itself.

By the end of the project a user can type www.google.com on a client PC, watch DNS resolve the name, see the TCP / TLS handshake, and finally see the index page of the web server - exactly the same flow that happens on a real browser.

1.1  Topology at a Glance

3 x client PCs  -  PC0, PC1, PC2  -  connected to a Cisco 2960-24TT switch.

1 x Cisco 2911  -  edge router, providing DHCP, the default route, and PAT.

1 x Cisco 2911  -  'Internet' router, simulating the ISP / public backbone.

1 x Server-PT  -  Google DNS, IP 8.8.8.8.

1 x Server-PT  -  Google Web Server, IP 8.8.8.10, hosting www.google.com.

1.2  IP Addressing Plan

| Device | Interface | IP Address | Subnet Mask | Notes |
| --- | --- | --- | --- | --- |
| Edge Router | G0/0 (LAN) | 192.168.1.1 | 255.255.255.0 | Default gateway for PCs |
| Edge Router | G0/1 (WAN) | 200.200.200.1 | 255.255.255.252 | Point-to-point to ISP |
| Internet Router | G0/0 (WAN) | 200.200.200.2 | 255.255.255.252 | Point-to-point to edge |
| Internet Router | G0/1 (Servers) | 8.8.8.1 | 255.255.255.0 | Where Google servers live |
| Google DNS | NIC | 8.8.8.8 | 255.255.255.0 | Authoritative for google.com |
| Google Web Server | NIC | 8.8.8.10 | 255.255.255.0 | Hosts www.google.com |
| PC0 | NIC | 192.168.1.10 | 255.255.255.0 | DHCP |
| PC1 | NIC | 192.168.1.11 | 255.255.255.0 | DHCP |
| PC2 | NIC | 192.168.1.12 | 255.255.255.0 | DHCP |

Why two /30s and two /24s?  Two point-to-point links each get their own /30 (two usable IPs, no waste). The LAN and the server segment both use /24 - plenty of room for clients, servers and future growth.

2.  Switch Configuration

The 2960-24TT is a plain Layer-2 access switch. No VLANs, no trunking - a single broadcast domain. The configuration sets a hostname and a console password only.

Switch>  enable

Switch#  configure terminal

Switch(config)#  hostname SW-Office

SW-Office(config)#  banner motd # Office LAN - Authorized Only #

SW-Office(config)#  line console 0

SW-Office(config-line)#  logging synchronous

SW-Office(config-line)#  password Console123

SW-Office(config-line)#  login

SW-Office(config-line)#  exit

SW-Office(config)#  service password-encryption

SW-Office(config)#  end

SW-Office#  write memory

3.  Edge Router  (Cisco 2911)

The edge router is the most complex device in the project. It has three jobs: (1) be the LAN default gateway, (2) hand out DHCP leases to the PCs, (3) translate all LAN-side private IPs to its single public WAN IP using PAT.

3.1  Interfaces

EdgeRouter>  enable

EdgeRouter#  configure terminal

EdgeRouter(config)#  hostname EdgeRouter

EdgeRouter(config)#  interface GigabitEthernet0/0

EdgeRouter(config-if)#  description ** LAN **

EdgeRouter(config-if)#  ip address 192.168.1.1 255.255.255.0

EdgeRouter(config-if)#  ip nat inside

EdgeRouter(config-if)#  no shutdown

EdgeRouter(config-if)#  exit

EdgeRouter(config)#  interface GigabitEthernet0/1

EdgeRouter(config-if)#  description ** WAN **

EdgeRouter(config-if)#  ip address 200.200.200.1 255.255.255.252

EdgeRouter(config-if)#  ip nat outside

EdgeRouter(config-if)#  no shutdown

EdgeRouter(config-if)#  exit

3.2  DHCP server  (running on the edge router)

EdgeRouter(config)#  ip dhcp excluded-address 192.168.1.1 192.168.1.20

EdgeRouter(config)#  ip dhcp pool OFFICE-LAN

EdgeRouter(dhcp-config)#  network 192.168.1.0 255.255.255.0

EdgeRouter(dhcp-config)#  default-router 192.168.1.1

EdgeRouter(dhcp-config)#  dns-server 8.8.8.8

EdgeRouter(dhcp-config)#  lease 7

EdgeRouter(dhcp-config)#  exit

3.3  Default route

EdgeRouter(config)#  ip route 0.0.0.0 0.0.0.0 200.200.200.2

3.4  NAT / PAT (overload)

EdgeRouter(config)#  access-list 1 permit 192.168.1.0 0.0.0.255

EdgeRouter(config)#  ip nat inside source list 1 interface GigabitEthernet0/1 overload

Access-list 1 matches every LAN-side IP. The 'overload' keyword enables PAT: many private IPs share the single public IP 200.200.200.1, distinguished by source port.

3.5  Hardening

EdgeRouter(config)#  banner motd # Edge Router - Internal Network #

EdgeRouter(config)#  enable secret Class123

EdgeRouter(config)#  line console 0

EdgeRouter(config-line)#  password Console123

EdgeRouter(config-line)#  login

EdgeRouter(config-line)#  exit

EdgeRouter(config)#  service password-encryption

EdgeRouter(config)#  end

EdgeRouter#  write memory

4.  Internet Router  (Cisco 2911)

The 'Internet' router is intentionally minimal. It owns the /30 to the edge router, the /24 where the Google servers live, and a single static return route so replies can find their way back to the LAN through the PAT table.

Internet>  enable

Internet#  configure terminal

Internet(config)#  hostname Internet

Internet(config)#  interface GigabitEthernet0/0

Internet(config-if)#  description ** LINK TO EDGE ROUTER **

Internet(config-if)#  ip address 200.200.200.2 255.255.255.252

Internet(config-if)#  no shutdown

Internet(config-if)#  exit

Internet(config)#  interface GigabitEthernet0/1

Internet(config-if)#  description ** GOOGLE SERVER LAN **

Internet(config-if)#  ip address 8.8.8.1 255.255.255.0

Internet(config-if)#  no shutdown

Internet(config-if)#  exit

Internet(config)#  ip route 200.200.200.0 255.255.255.252 200.200.200.1

Internet(config)#  end

Internet#  write memory

5.  Server Configuration

5.1  Google DNS  (8.8.8.8)

On the Google DNS server:

Set IP 8.8.8.8 / 255.255.255.0, gateway 8.8.8.1.

Open the Services tab > DNS, toggle the service ON.

Add a resource record: Name = www.google.com, Type = A Record, Address = 8.8.8.10.

After saving, any client that sends a DNS query for www.google.com to 8.8.8.8 will receive 8.8.8.10 as the answer.

5.2  Google Web Server  (8.8.8.10)

On the Google Web Server:

Set IP 8.8.8.10 / 255.255.255.0, gateway 8.8.8.1.

Open the Services tab > HTTP and HTTPS, toggle both ON.

Edit the default index.html so it reads 'Welcome to Google' - the browser will display this page.

Packet Tracer simulates a successful 200 OK response for both HTTP (port 80) and HTTPS (port 443).

6.  End-Device Configuration

All three PCs are DHCP clients. The pool is configured on the edge router, so each PC only needs to set IP Configuration to DHCP. The router hands out 192.168.1.10, .11 and .12 respectively, with mask 255.255.255.0, gateway 192.168.1.1 and DNS 8.8.8.8.

7.  End-to-End Test  -  Browser to www.google.com

From PC0, open Desktop > Web Browser and enter: http://www.google.com

7.1  Step 1  -  DNS resolution

The browser asks the configured DNS server (8.8.8.8) for the A record of www.google.com. The query leaves the PC, hits the default gateway 192.168.1.1, the edge router translates the private source IP to 200.200.200.1 (PAT) and forwards the packet to 8.8.8.8. The DNS server replies with 8.8.8.10. The browser caches the answer and moves on.

PC0>  nslookup www.google.com

Server:    8.8.8.8

Address:   8.8.8.8#53

Non-authoritative answer:

Name:      www.google.com

Address:   8.8.8.10

7.2  Step 2  -  TCP three-way handshake

The browser opens a TCP connection to 8.8.8.10 port 80. Three packets (SYN, SYN+ACK, ACK) traverse the same path: PC  ->  edge router (PAT source = 200.200.200.1)  ->  Internet router  ->  web server. The web server replies to 200.200.200.1; the edge router uses its PAT table to map the destination back to the original private PC IP, and the PC receives the SYN+ACK.

7.3  Step 3  -  HTTP request and response

The browser sends GET / HTTP/1.1 to 8.8.8.10. The web server answers with the index.html page. Inside PC0's browser window the 'Welcome to Google' page appears - proof that the entire chain (DHCP, routing, NAT/PAT, DNS, TCP, HTTP) is working end-to-end.

7.4  Same test with HTTPS

Change the URL to https://www.google.com. The browser still resolves DNS, opens a TCP connection, performs a TLS handshake with the server, and sends the encrypted HTTP request. Packet Tracer shows a successful connection - visually identical to the HTTP case - confirming that HTTPS (port 443) is also handled by the web server.

8.  Verification Commands

8.1  NAT translation table

EdgeRouter#  show ip nat translations

Pro  Inside global      Inside local       Outside local      Outside global

tcp  200.200.200.1:1024 192.168.1.10:1024  8.8.8.10:80        8.8.8.10:80

8.2  DHCP bindings

EdgeRouter#  show ip dhcp binding

IP address       Client-ID         Lease expiration        Type

192.168.1.10     00E0.A38C.ABCD    --                      Automatic

192.168.1.11     00E0.A38C.ABCE    --                      Automatic

192.168.1.12     00E0.A38C.ABCF    --                      Automatic

8.3  Routing table

EdgeRouter#  show ip route

Codes: C - connected, S - static, ...  Gateway of last resort is 200.200.200.2

S*    0.0.0.0/0 [1/0] via 200.200.200.2

C     192.168.1.0/24 is directly connected, GigabitEthernet0/0

C     200.200.200.0/30 is directly connected, GigabitEthernet0/1

9.  Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| PC stuck on 169.254.x.x | DHCP pool misconfigured or no default route | Re-check 'default-router 192.168.1.1' and 'ip route 0.0.0.0 0.0.0.0 200.200.200.2' |
| Browser shows 'host unreachable' | NAT/PAT not enabled or 'ip nat inside' missing on the LAN interface | Add 'ip nat inside' on G0/0 and 'ip nat outside' on G0/1 |
| DNS resolves but HTTP request fails | Web server HTTP/HTTPS service is OFF, or wrong gateway | Toggle HTTP and HTTPS ON; verify server gateway 8.8.8.1 |
| Web server unreachable from PC | Internet router missing return route | Add 'ip route 200.200.200.0 255.255.255.252 200.200.200.1' |

10.  Physical Layer

The same scenario is also shown in Physical view. The Corporate Office rack contains the edge router, the Internet router, the Google DNS server, the Google Web Server, the access switch, and three patch panel ports. Each PC sits at its own desk in the office space. Cables run physically from the patch panel to the switch ports and from there to the rack.

11.  Conclusion

Project 03 ties together every concept from the previous two projects and adds the missing link to a real Internet service: DNS, NAT/PAT, and a server hosting a real (simulated) web page. From the user's point of view, 'going to www.google.com' looks like a single click. From the network's point of view it is: DHCP lease  ->  default gateway  ->  DNS query  ->  NAT / PAT  ->  WAN routing  ->  TCP handshake  ->  TLS handshake  ->  HTTP request  ->  HTTP response  ->  reverse NAT  ->  browser. Building this project once, end-to-end, is the best way to make every one of those steps concrete.

Amirali Gholian  -  Network Lab  -  Cisco Packet Tracer